FEMS 2897110

Region 4A

Zone 1

FAC ID:  PA035

POC: (AFOS) Cindy Croyle 412-477-7816   (RFOS) Scott Kawski 814-392-6879

Priority- Routine

Description of Repair:  Repair two exterior doors.  Replace Front Door - Remove and dispose of the exterior French style 32” double door. Approximate opening 64” wide x 82” high, double door, frame, hardware, threshold, and any associated components completely. To include the Transom structure located immediately above the door frame.

**NOTE** There is a card reader at this location. Care shall be utilized during demolition as this, system shall be reinstalled.  Install a new set of double exterior doors to the opening. The new double doors shall be different in size. As you stand outside the door on the right shall be 36” wide x 82” high with the door on the left measuring 28” wide x 82” high.

•  The new 36” door shall have a Vision Lite window measuring 5” W x 20”H and be wire reinforced.

•  Replace double door frame.

•  Replace threshold.

•  Replace Lockable Removable “Mullion”.

Replace (2) stainless steel continuous hinges.

(Replace 2) surface mounted door closers.

Replace (2) kick plates.

•  Reinstall the card reading mechanism.

•  Ensure the door functions as required to include the card reader and/or key function properly with the panic hardware.

Replace Handicap Side Door - remove and dispose of the exterior French style 30” double door.  Approximate opening 60” wide x 82” high, double door, frame, hardware, threshold, and any associated components completely. To include the Transom structure located immediately above the door frame.

**NOTE** There is a card reader at this location. Care shall be utilized during demolition as this system shall be reinstalled.

Install - a new set of double exterior doors to the opening. The new double doors shall be different in size. As you stand outside the door on the right shall be 36” wide x 82” high with the door on the left measuring 24” wide x 82” high. • The new 36” door shall have a Vision Lite window measuring 5” wide x 20” high and be wire reinforced.

• Replace double door frame.

• Replace threshold.

• Replace Lockable Removable “Mullion”.

Replace (2) stainless steel continuous hinges.

Replace (2) surface mounted door closers.

Replace (2) kick plates

Reinstall the card reading mechanism.

• This door shall have a Rain Deflector installed above the door

-Type of Funding: (COR input)

-Paying Activity Code: (COR input)

-Date & Time: 10/18/2024 0930 hrs

-Associated Asset ID: PA035

Recommend:   Marinaro Electric