| CMI Cost Estimate |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Funding Request |  |  |  |  |  |  |  |  |
| NY128 |  |  |  |  |  |  |  |  |
| Only Handicap toilet in men's room |  |  |  |  |  |  |  |  |
| CSS #25071 - CMI # 8233 |  |  |  |  |  |  |  |  |
| Prepared By: J. Bayne |  |  |  |  |  |  |  |  |
| 05.18.20 |  |  |  |  |  |  |  |  |
|  |  |  | Labor |  |  | Sub | OHP |  |
| Description | Rate | Hours | Cost | Material | Tax | Total | 20% | Total |
| 1. Repair Cost |  |  |  |  |  |  |  |  |
| 1.1 Labor | $ 120.00 | 8 |  |  |  |  |  | $ 960.00 |
| 1.2 Material |  |  |  |  | $ - | $ - | $ - | $ 697.17 |
| 1.3 Misc |  |  |  |  |  |  |  | $331.43 |
| Sub Total Repair Cost |  |  |  |  |  |  |  | $ 1,988.60 |
| Grand Total |  |  |  |  |  |  |  | $ 1,988.60 |

Only Handicap toilet in men's room on first floor constantly flushes. Water is currently shut off because toilet would not stop running. Water also leaking from flush valve. 1st sink on left in same men's room will not come on. Suspect that sensor is malfunctioning.

Proposal to:

- Rebuild (1) commode flushometers

- Disassemble and clean mineral deposits from valve body.

- Install Repair/seal & diaphragm kits

- Install vacuum breaker assemblies

- Install brass spud on water closet

- Replace el595 sensor