CSS 95258

Region 4A

Zone 2

FAC ID: PA118

POC: (AFOS) Cindy Croyle 412-477-7816 (RFOS) Scott Kawski 814-392-6879

Priority- Urgent

Description of Repair:  Warehouse - repair standing seam metal roofing system. There is a possible leak approximately 40 feet from the west wall on the ridge vent.  Rubber boots around vents are deteriorating and need to be replaced. Gutters need to be cleaned out, repair seams, rivets, etc.. and seal.  AMSA - has two roof systems, first one is rubber roofing and the second is standing seal metal.  Membrane is pulling away from rake edge on the east side of the building and appears to be leaking along the exterior wall.  Roofing will need to be pulled back and any damage to the insulation, blocking and rake edge metal (rust debris, etc., removed/cleaned) will need to be repaired.  Several seams and patches will need to be repaired that are breaking down.  Rubber boots for vents are missing metal straps or pulled away, splitting and not sealed properly, will need to be replaced/repaired.  Gutters are the same as warehouse and will need to be repaired.  Pictures attached for review.  This would be a temporary repair until a full roof replacement project can be developed.

-Type of Funding: (COR input)

-Paying Activity Code: (COR input)

-Date & Time: 10/5/2023 0800 hours

-Associated Asset ID: PA118

Suggested/Reccomended Vendor: