Main exit gate arm swings open when vehicle from outside the facility approaches.

** Diagnostic: The issue is that the exit loop goes under the gate arm for safety purposes so that the gate arm does not come down on a car. Unfortunately, leadership is allowing them to pull in through the exit bound gate and opening the gate operator.

If you would like to correct this issue, we will need to move the exit loop back and install a second safety loop on the entrance side of the outbound lane.

Proposal to: Move Exit Loop Back, and Install Second Safety Loop

| CMI Cost Estimate |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Over & Above |  |  |  |  |  |  |  |  |
| NY116 |  |  |  |  |  |  |  |  |
| Main exit gate arm swings open |  |  |  |  |  |  |  |  |
| CSS #89644 - CMI #19228 |  |  |  |  |  |  |  |  |
| Prepared By: J. Bayne |  |  |  |  |  |  |  |  |
| 01.24.23 |  |  |  |  |  |  |  |  |
|  |  |  | Labor |  |  | Sub | OHP |  |
| Description | Rate | Hours | Cost | Material | Tax | Total | 20% | Total |
| 1. Repair Cost |  |  |  |  |  |  |  |  |
| 1.1 Labor | $ 85.00 | 32 |  |  |  |  |  | $ 2,720.00 |
| 1.2 Material |  |  |  |  | $ - | $ - | $ - | $ 361.88 |
| 1.3 Saw cutting machine |  |  |  |  | $ - | $ - | $ - | $ 250.00 |
| Sub Total Repair Cost |  |  |  |  |  |  |  | $ 3,331.88 |
| 2. Initial Service Call Investigation |  |  |  |  |  |  |  |  |
| 2.1 CE Factor 1.02% |  |  |  |  |  |  |  | $ 66.63 |
| Grand Total |  |  |  |  |  |  |  | $ 3,398.51 |