Saugerties, NY USARC

Access Control System Replacement

DEMO

Contractor shall remove entire Access Control Headend Equipment to include Server, Enrollment Station and UPS Battery Backup in preparation for replacement.

Contractor shall demo (3) existing Access Control Panels. One Access Control Panel in Main Training Building, One in Cold Storage Building and one in OMS Building.

Demo shall include electronics and cabinetry in preparation for new Cabinets.

Contractor shall demo a total of (7) Readers throughout entire facility in preparation for replacement Readers.

INSTALL SERVICES

Contractor shall install new Desktop Server inclusive of Enrollment Station, Monitor and Veridt Bio Guest Pass 2 at existing location.

Contractor shall install (3) new Galaxy Access Control Cabinets where existing cabinetry was removed.

Contractor shall rewire and reconfigure wiring for Galaxy Access Control Cabinetry inclusive of Veridt Readers at each existing location.

Contractor shall utilize all existing Door Locking Hardware and Power Supplies at each location.

Contractor shall install (7) new Readers at existing locations where removed during demo. Readers shall be Veridt Biometric Stealth Multimode equipped with Guest Pass 2.

Contractor shall network, configure and test prior to demonstrations with end-user. 99th Division RFOS shall coordinate with Facility proper Administrators prior to demonstration.

Contractor shall demonstrate to Administrators of the System for a period of no more than 8 hours at the completion of the job.

Contractor shall be both Galaxy and Veridt certified for these efforts and if requested, shall provide proof of such.